BAT-3HS

HDMI to 3G/HD/SD-SDI Converter

(1) 기 능

본 기기는 HDMI 신호를 3G/HD/SD-SDI 신호로 변환하는 컨버터 장비로서, 최저의 SDI Jitter성능을 갖고 있어 뛰어난 신호품질을 제공하여 사용자가 잡음이 들어가있는 아날로그 비디오/오디오 신호를 SDI케이블을 통해서 멀리 좋은 품질로 보낼 수 있어야 하고 다음과 같은 사양에 준한다.

- 효율적인 열 발산설계

- 타 브랜드와 호환성

- 다양한 오디오 샘플링 지원(32~192kHz to 48kHz)

- 다양한 입력전원 지원(+7V~+15V)

- 저전력 소모(3Watt Max.)

- 비디오 모드 자동 검색 - 3G/HD/SD

- 별도의 Analog Stereo Audio (RCA) 입력 선택 가능

- HDMI ｖ1.4 지원

- SDI Output x 2 지원 [ BNC 75 Ohm ]

(2) 기기 특성

- Supported Video Formats: 720(1440) x 480i @ 59.94/60, 720(1440) x 576i @ 50

1280x720p@50,1280x720p@59.94/60

1920x1080p@23.94/24,1920x1080p@25,

1920x1080p@29.97/30

1920x1080p@50,1920x1080p @ 59.94/60

- Input: HDMI with embedded audio & Auto mute

when DVI input

- Outputs: SMPTE-259/272/274/292/296/299/424/425

3G/HD/SD-SDI, 2 x BNC

- Supports 8 audio channels (4 stereo pairs)

: when the input audio sample rate is 48kHz

- Supports Audio Sample Rate Converter Mode

: when the input audio sample rate is not 48kHz

- Supports 2 audlochannels (1 stereo pair)

- Auto detection of the input audio sample rate

- Auto switching audio sample rate converter mode

- Supports arbitrary input audio sample rate (32kHz -192kHz)

- Output audio sample rate is 48kHz

- BAT-3HS 및 동등 이상 품이어야 한다.