BAT-SA

HD/SD-SDI to Analog Converter

(1) 기 능

본 기기는 HD/SD-SDI 신호를 Analog 신호로 변환하는 컨버터 장비로서, 최저의 SDI Jitter성능을 갖고 있어 뛰어난 신호품질을 제공하여 사용자가 잡음이 들어가있는 아날로그 비디오/오디오 신호를 케이블을 통해서 멀리 좋은 품질로 보낼 수 있어야 하고 다음과 같은 사양에 준한다.

- 효율적인 열 발산설계

- 타 브랜드와 호환성

- 다양한 오디오 샘플링 지원(32~192kHz to 48kHz)

- 다양한 입력전원 지원(+7V~+15V)

- 저전력 소모(3Watt Max.)

- Re-Clocked Loop-through HD/SD-SDI Output [ BNC 75 Ohm ]

- Component 또는 Composite 출력 선택 가능 [ BNC 75Ohm ]

- 2개의 오디오 채널 지원 (1 Stereo Pairs)

(2) 기기 특성

- Supported Video Formats: 720(1440) x 480i @ 59.94/60, 720(1440) x 576i @ 50

1280x720p @ 50, 1280x720p @ 59.94/60

1920 x 1080i @ 50, 1920 x 1080i @ 59.94/60

1920x1080p @ 23.94/24,1920x1080p @ 25,

1920 x 1080p @ 29.97/30

- Video Input: HD and SD-SDI (auto-selected),

SMPTE-259/272/274/292/296/299,

1x BNC input, 1x BNC Loop-through output

- Video output: Selectable Component(YPbPr) or

Composite(CVBS and S-Video) by DIP Switch

NTSC, PAL support 3x BNC output

- Audio Outputs: 2 channels BNC - style analog outputs at -10dBV

- Input Power Range: +7V ~ +15V

- Power Consumption: 3Watt [Max]

- Power Adapter: Input : 100V ~ 240V, 50/60Hz

Output : +12V DC, 2.0A

- BAT-SA 및 동등 이상 품이어야 한다.