HDMI/SDI Cross Converter with Audio Mux/Demux

Ez-MD+

(1) 기 능

본 기기는 SDI/HDMI 신호를 지원하는 크로스컨버터로써

입력 - SDI 1개, HDMI 1개  출력 - SDI 1개, HDMI 1개 로 구성되어야하며

Analog Audio(XLR)입출력을 지원하는 장비임

- SD/HD/3G 지원

- 전용 케이블을 이용하여 XLR 오디오 입출력을 지원해야함

- 딥스위치를 통해 출력 해상도를 설정 가능하여야함

- 로터리스위치를 통해 오디오 딜레이를 설정 가능하여야함

- LED를 통해 입력신호의 인식여부를 판단 가능하여야함

- 딥스위치를 통해 HDMI, SDI 입력 모드 설정이 가능하여야함

- 부착 가능한 별도의 브라켓 제공

(2) 기기 특성

Data Input

SDI Video Input: SD/HD/3G-SDI (BNC 75Ω) x1

HDMI Video Input: SD/HD/3G (HDMI Type A) x1

Reference Input: B.B , TRI (BNC 75Ω) x1

Audio Input: D-SUB 9PIN (Female) x1

Video Formats

- 720x480i@59.94/60

- 720x576i@50

- 1280x720p@50/59.94/60

- 1920x1080i@50/59.94/60

- 1920x1080p@23.98/24/25/29.97/30

- 1920x1080p@50/59.94/60

Data Output

SDI Video Output: SD/HD/3G-SDI (BNC 75Ω) x1

HDMI Video Output: SD/HD/3G (HDMI Type A)

x1

Audio Output : D-SUB 9PIN (Female) x1

Video Formats

- 720x480i@59.94 (DIP SW : 1111)

- 720x576i@50 (DIP SW : 0111)

- 1280x720p@50 (DIP SW : 1011)

- 1280x720p@59.94 (DIP SW : 0011)

- 1280x720p@60 (DIP SW : 1101)

- 1920x1080i@50 (DIP SW : 0101)

- 1920x1080i@59.94 (DIP SW : 1001)

- 1920x1080i@60 (DIP SW : 0001)

- 1920x1080p@23.98 (DIP SW : 1110)

- 1920x1080p@24 (DIP SW : 0110)

- 1920x1080p@25 (DIP SW : 1010)

- 1920x1080p@29.97 (DIP SW : 0010)

- 1920x1080p@30 (DIP SW : 1100)

- 1920x1080p@50 (DIP SW : 0100)

- 1920x1080p@59.94 (DIP SW : 1000)

- 1920x1080p@60 (DIP SW : 0000)

Connections

Power : DC Power Input (DC Jack) x1

Update : Debug (Micro-USB) x1

DIP Switch : (10 Pole) x1

Video Output Resolution, Video Output Ratio, Video

Input(SDI or HDMI),Audio(INT or EXT), Sync Mode :

Free Running, Input lock, Genlock (B.B or TRI-Level),

Audio Gain(0 or -10dB)

ROTARY Switch : (10 Step) x1

Audio Delay Control (50ms Step)

Display

Link (GREEN x1) : HDMI or SDI Input Video Detect

Ref (GREEN x1) : REF Input Signal Detect

Left (GREEN x1, RED x1) : INPUT AUDIO Level

Meter(Left)

Right (GREEN x1, RED x1) : INPUT AUDIO Level

Meter(Right)

Electrical

Power Input: DC +11V to +13V

Power Consumption: 8W(max.)

Operation Temperature: 0~45℃

Physical

Dimension: 130x100.4x26.9 (include connector)

Weight: 0.4 kg(typ.)

ez-MD+

- ez-MD+ 및 동등 이상 품이어야 한다.