SDI to HDMI Converter with Display and Scaler

Ez-SHV+

(1) 기 능

본 기기는 SDI 신호를 HDMI 신호로 변환하는 컨버터 장비임.

SDI 입력신호를 사용하여 원하는 해상도로 출력되도록 설계 가능

- 딥스위치를 통해 다양한 출력 해상도 설정

- 오디오 DE-Enbedded 기능 지원

- 비디오 모드 자동 검색 - 3G/HD/SD

- SDI Output x 2 지원 [ BNC 75 Ohm ]

- 부착 가능한 별도의 브라켓 제공

- 전면 LCD를 통해 입력신호 확인 및 정보 파악 가능

(2) 기기 특성

ez-converter(ez-SHV+)

Data Input

SDI Video Input: SD/HD/3G (BNC 75Ω) x1

Video Formats: - 720x480i@59.94/60 - 720x576i@50 - 1280x720p@50/59.94/60 - 1920x1080i@50/59.94/60 - 1920x1080p@23.94/24/25/29.97/30 - 1920x1080p@50/59.94/60

Data Output

SDI LOOP Output: SD/HD/3G-SDI (BNC 75Ω) x2 Analog Audio output: Stereo Jack x1 HDMI Video Output : SD/HD/3G (HDMI Type A)

Video Formats: - 720x480i@59.94 (DIP SW : 1111) - 720x576i@50 (DIP SW : 0111) - 1280x720p@50 (DIP SW : 1011) - 1280x720p@59.94 (DIP SW : 0011) - 1280x720p@60 (DIP SW : 1101) - 1920x1080i@50 (DIP SW : 0101) - 1920x1080i@59.94 (DIP SW : 1001) - 1920x1080i@60 (DIP SW : 0001) - 1920x1080p@23.98 (DIP SW : 1110) - 1920x1080p@24 (DIP SW : 0110) - 1920x1080p@25 (DIP SW : 1010) - 1920x1080p@29.97 (DIP SW : 0010) - 1920x1080p@30 (DIP SW : 1100) - 1920x1080p@50 (DIP SW : 0100) - 1920x1080p@59.94 (DIP SW : 1000) - 1920x1080p@60 (DIP SW : 0000)

Connections

Power : DC Power Input (DC Jack) x1

DIP Switch : (6 Pole) x1 Video In/Out Resolution, Video Output Ratio, Audio(INT. or EXT.)

MODE : (Push Switch) x1 LCD ON/OFF, Menu Selection

Display

2.7 inch TFT LCD : Input Video Display, In/Output Video Resolution Display, Input Audio Level Meter Display

Electrical

Power Input: DC +5V to 18V

Power Consumption: 3W(max.)

Operation Temperature: 0~45℃

Physical

Dimension: 86.3x57.2x26.6 (include connector)

Weight: 0.3 kg(typ.)

- ez-SHV+ 및 동등 이상 품이어야 한다.